Safe Work Method Statement – Work carried out in or near a confined space

| Organisation Details |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Organisation Name: |  |  |  | Contact Name: |  |  |
| ACN/ABN |  |  |  | Contact Position: |  |  |
| Address: |  |  |  | Contact Phone No: |  |  |
| Activity: Work carried out in or near a confined space |  |  |  | This SWMS has been developed in consultation with: Reviewed by: Position: Date: |  |  |
| Job Address: |  |  |  |  |  |  |
| Equipment Used: | Ventilation Equipment, Rescue Equipment, Gas Detector, Portable Tools |  |  |  |  |  |
| Work Health Safety & Environmental Legislation | Work Health & Safety Act 2011 Work Health & Safety Regulation 2017 |  | Codes or Standards applicable to work |  | Australian Standard 3500 National Plumbing and Draining Australian Standard 5601 Gas Installations NSW Plumbing Act 2011 and Regulations 2017 (replacing the NSW Code of Practice for Plumbing and Drainage Regulation) Plumbing Code of Australia (PCA) Confined spaces Hazardous manual tasks How to manage and control asbestos in the workplace How to manage work health and safety risks Managing the risk of falls at workplaces How to safely remove asbestos Labelling of workplace hazardous chemicals Managing noise and preventing hearing loss at work Managing the work environment and facilities Preparation of safety data sheets for hazardous chemicals Work health and safety consultation, coordination and cooperation |  |

| Level | Description of Consequence or Impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood/Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

## Work Method Statement

Activity: Work carried out in or near Confined Spaces

| Task | Risk | Risk Score | Control | Person Responsible |
| --- | --- | --- | --- | --- |
| Prior to entering a confined space |  |  | Determine whether a space is a confined space. Refer to Confined Spaces Code of Practice Complete Confined Spaces Entry Permit All Workers to be trained and consulted on emergency rescue procedures. Ensure for the entire period the confined space permit is valid, a stand by person is required to monitor the workers and the confined space. |  |
| Entry and exit from Confined Space | Slips/Trips/Falls | 2 | Use of fixed ladders, platforms, and walkways for access/exit Ensure Access points are unobstructed |  |
|  | Unsafe Oxygen Level | 1 | Atmospheric testing to be carried out to determine exposure levels |  |
| Task | Risk | Risk Score | Control | Person Responsible |
| Working in Confined Space | Unsafe Oxygen Levels | 1 | Air monitoring to be carried out by a competent person using a suitable, correctly calibrated gas detector. Atmosphere to be monitored for oxygen content, airborne concentration of flammable contaminants and airborne concentration of potentially harmful contaminants. Ensure adequate ventilation (natural or mechanical) |  |
|  | Fire and Explosion Engulfment | 1 | Ensure flammable gas, vapour or mist does not exceed 5% of its lower explosive limit (LEL) Avoid any Hot Work. Ensure Electrical equipment is tested and tagged in accordance with AS/NZS 3012:2010 (Electrical installations—Construction and demolition sites) |  |
|  | Poor Lighting | 3 | Ensure suitable lighting (natural or mechanical) |  |
|  | Noise | 3 | Workers to wear ear muffs/ear plug |  |
| Task | Risk | Risk Score | Control | Person Responsible |
| Working in Confined Space (continued) | Untidy work areas | 3 | Work areas and passage ways to remain clear at all times Rubbish and materials are to be removed on an ongoing basis |  |
|  | Fumes | 3 | Ensure adequate ventilation Workers must wear dust mask or respirator. Constant Air monitoring to be carried out by a competent person using a suitable, correctly calibrated gas detector. Atmosphere to be monitored for oxygen content, airborne concentration of flammable contaminants and airborne concentration of potentially harmful contaminants. |  |
|  | Drowning/Flowing Water | 1 | Isolate services. Ensure services are locked out and tagged “danger do not use” |  |
|  | Unauthorised Access | 2 | Stand by Person to monitor entry/exit points to confined space. Signage to be installed to warn against entry by other unauthorised people. Barricade Work Area |  |

| This SWMS has been developed through consultation with our workers and has been read, understood and signed by all workers undertaking the works: |  |  |
| --- | --- | --- |
| Print Names: | Signatures: | Dates: |

| Review No | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial: |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |