Safe Work Method Statement – Excavation of 1.5mtr or Greater

| Organisation Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Organisation Name: |  |  | Contact Name: |  |  |
| ACN/ABN |  |  | Contact Position: |  |  |
| Address: |  |  | Contact Phone No: |  |  |
| Activity: Excavation of 1.5mtr or Greater |  |  | This SWMS has been developed in consultation with: Reviewed by: Position: Date: |  |  |
| Job Address: |  |  |  |  |  |
| Equipment Used: | Excavator, Backhoe, Portable Tools |  |  |  |  |
| Work Health Safety & Environmental Legislation | Work Health & Safety Act 2011 Work Health & Safety Regulation 2017 | Codes or Standards applicable to work |  | Australian Standard 3500 National Plumbing and Draining Australian Standard 5601 Gas Installations NSW Plumbing Act 2011 and Regulations 2017 (replacing the NSW Code of Practice for Plumbing and Drainage Regulation) Plumbing Code of Australia (PCA) Confined spaces Hazardous manual tasks How to manage and control asbestos in the workplace How to manage work health and safety risks Managing the risk of falls at workplaces How to safely remove asbestos Labelling of workplace hazardous chemicals Managing noise and preventing hearing loss at work Managing the work environment and facilities Preparation of safety data sheets for hazardous chemicals Work health and safety consultation, coordination and cooperation |  |

| Level | Description of Consequence or Impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood/Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

## Work Method Statement

Activity: Excavation of 1.5mtr or Greater

| Task | Risk | Risk Score | Control | Person Responsible |
| --- | --- | --- | --- | --- |
| Establishment of Site Prepare to Excavate | Contact with overhead or underground services resulting in electrocution/Electric Shock, Fire/ Explosion | 1 | Underground Essential Services to be located prior to commencement of work. Dial-before-you-dig 1100 or www.dialbeforeyoudig.com.au Use location equipment to locate buried services Work only within safe distances of power lines as published by the local supply authority. Isolate any services (electricity, gas) before excavation commences No work is to be carried out on live services. If service is unable to be isolated contact the relevant supply authority Check stability of ground before positioning machinery (including vehicles) |  |
| Working in Excavation | Trench Collapse | 2 | Trench is to be benched or shored where excavation is 1.5mtr or greater. |  |
|  | Contact with overhead or underground services (Electrocution/Electric Shock Fire Explosion) | 1 | Work only within safe distances of power lines as published by the local supply authority. Isolate any services (electricity, gas) before excavation commences No work is to be carried out on live services. If service is unable to be isolated contact the relevant supply authority |  |
| Task | Risk | Risk Score | Control | Person Responsible |
| Working in Excavation (continued) | Working in vicinity of mobile powered plant | 2 | Carry out all pre-inspection checklists and check log books. Ensure operators of machinery are competent Exclusion zones to be set out and clearly identified. Barricade work area. Hi-Visibility clothing to be worn Workers to avoid working in the ‘swing zone’ of heavy machinery |  |
|  | Falls Objects Falling | 2 | Exclusion zones to be set out and clearly identified. Barricade area. Ensure tools and materials are secured Workers to wear hard hats |  |
|  | Dust | 3 | Equipment used to minimise dust suppression or workers must wear dust mask or respirator. |  |
|  | Noise | 3 | Workers to wear ear muffs/ear plugs |  |
|  | Fumes | 3 | Ensure adequate ventilation Workers must wear dust mask or respirator. |  |
|  | Untidy work areas | 3 | Work areas and passage ways to remain clear at all times Rubbish and excavated material are to be removed on an ongoing basis |  |
|  | Access/Egress from trench | 2 | Ladders, steps or ramp access to be provided and maintained to ensure entry/exit into trench |  |
|  | Unauthorised Access | 2 | Any open excavation when not being worked on is to be covered and secured to prevent unauthorised access |  |
| Task | Risk | Risk Score | Control | Person Responsible |
| Reinstatement | Slips/Trips/Falls | 3 | Ground is to be fully reinstated after completion of excavation works. All excess material to be disposed of correctly |  |

| This SWMS has been developed through consultation with our workers and has been read, understood and signed by all workers undertaking the works: |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Print Names: |  |  |  | Signatures: |  |  |  | Dates: |  |  |  |  |
| Review No | 01 | 02 | 03 |  | 04 | 05 | 06 |  | 07 | 08 | 09 |  |
| Initial: |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |