Safe Work Method Statement – Carried out on or near Pressurised Gas

| Organisation Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Organisation Name: |  |  | Contact Name: |  |  |
| ACN/ABN |  |  | Contact Position: |  |  |
| Address: |  |  | Contact Phone No: |  |  |
| Activity: Work carried out on or near pressurised gas |  |  | This SWMS has been developed in consultation with: Reviewed by: Position: Date: |  |  |
| Job Address: |  |  |  |  |  |
| Equipment Used: | Portable tools, Oxy/Acet, Mapp Gas |  |  |  |  |
| Work Health Safety & Environmental Legislation | Work Health & Safety Act 2011 Work Health & Safety Regulation 2017 | Codes or Standards applicable to work |  | Australian Standard 3500 National Plumbing and Draining Australian Standard 5601 Gas Installations NSW Plumbing Act 2011 and Regulations 2017 (replacing the NSW Code of Practice for Plumbing and Drainage Regulation) Plumbing Code of Australia (PCA) Confined spaces Hazardous manual tasks How to manage and control asbestos in the workplace How to manage work health and safety risks Managing the risk of falls at workplaces How to safely remove asbestos Labelling of workplace hazardous chemicals Managing noise and preventing hearing loss at work Managing the work environment and facilities Preparation of safety data sheets for hazardous chemicals Work health and safety consultation, coordination and cooperation |  |

| Level | Description of Consequence or Impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood/Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

## Work Method Statement

Activity: Work carried out on or near pressurised Gas

| Task | Risk | Risk Score | Control | Person Responsible |
| --- | --- | --- | --- | --- |
| Establishment of Site Access Area | Contact underground services resulting in electrocution/Electric Shock, Fire/ Explosion | 1 | Underground Essential Services to be located prior to commencement of work. Dial-before-you-dig 1100 or www.dialbeforeyoudig.com.au Use location equipment to locate buried services. |  |
|  | Unauthorised access | 2 | Exclusion zones to be set out and clearly identified. Barricade work area |  |
| Test and Purge Gas Line | Fire/Explosion | 1 | Isolate gas supply. If unable to isolate gas supply contact local gas authority to arrange for a temporary disconnection. No work is to be carried out on live gas. Test service before commencement with pressure gauge. Define and make safe purge area and ensure no ignition sources are present. Purge outlet should be a minimum of 6 metres away from any ignition source No smoking in or near purge area |  |
| Excavation | Contact underground services resulting in electrocution/Electric Shock, Fire/ Explosion | 2 | Underground Essential Services to be located prior to commencement of work. Dial-before-you-dig 1100 or www.dialbeforeyoudig.com.au Use location equipment to locate buried services. Identify location of gas mains to determine if they are within work area. Only use hand digging techniques to expose main. Refer to SWMS Excavation |  |
| Task | Risk | Risk Score | Control | Person Responsible |
| Live Gas | Contact underground services resulting in electrocution/Electric Shock, Fire/ Explosion | 1 | Isolate gas supply. If unable to isolate gas supply contact local gas authority to arrange for a temporary disconnection. No work is to be carried out on live gas. |  |
| Hot Work | Fire/Explosion | 1 | Wear appropriate clothing that protects skin and is low flammability, remove any jewellery that may conduct heat and burn skin such as rings, necklaces. Flash back arresters to be fitted to oxy acetylene equipment Agree to an evacuation and communication plan in the event of fire or explosion Identify ALL combustible material surrounding work area Clear combustible material at least 3m away from work area and/or keep area saturated with water for at least 3m around the work. This may require a leaking pipe to be recharged to obtain water. Use fireproof shields (barrier to control sparks) to cover the combustible material. Have a fully charged 9litre fire extinguisher at the ready On completion of the work inspect the site and collect any hot material Hot waste pieces and items being worked on are to be cool before leaving the site. If possible actively cool with water. Collect small hot pieces in a metal container Use gloves, eye protection |  |
| Reinstate Gas Supply | Fire/Explosion | 1 | Test service with pressure gauge. Define and make safe purge area and ensure no ignition sources are present. Purge outlet should be a minimum of 6 metres away from any ignition source No smoking in or near purge area Comply to manufacturer’s instructions when commissioning appliances |  |

| This SWMS has been developed through consultation with our workers and has been read, understood and signed by all workers undertaking the works: |  |  |
| --- | --- | --- |
| Print Names: | Signatures: | Dates: |

| Review No | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial: |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |