Safe Work Method Statement – Pre Treatment Waste Services

| Organisation Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Organisation Name: |  |  | Contact Name: |  |  |
| ACN/ABN |  |  | Contact Position: |  |  |
| Address: |  |  | Contact Phone No: |  |  |
| Activity: Pre Treatment Waste Services |  |  | This SWMS has been developed in consultation with: Reviewed by: Position: Date: |  |  |
| Job Address: |  |  |  |  |  |
| Equipment Used: | Barriers, Excavator, Portable Tools |  |  |  |  |
| Work Health Safety & Environmental Legislation | Work Health & Safety Act 2011 Work Health & Safety Regulation 2017 | Codes or Standards applicable to work |  | Australian Standard 3500 National Plumbing and Draining Australian Standard 5601 Gas Installations NSW Plumbing Act 2011 and Regulations 2017 (replacing the NSW Code of Practice for Plumbing and Drainage Regulation) Plumbing Code of Australia (PCA) Confined spaces Hazardous manual tasks How to manage and control asbestos in the workplace How to manage work health and safety risks Managing the risk of falls at workplaces How to safely remove asbestos Labelling of workplace hazardous chemicals Managing noise and preventing hearing loss at work Managing the work environment and facilities Preparation of safety data sheets for hazardous chemicals Work health and safety consultation, coordination and cooperation |  |

| Level | Description of Consequence or Impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood/Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

## Work Method Statement

Activity: Pre Treatment Waste Services

| Task | Risk | Risk Score | Control | Person Responsible |
| --- | --- | --- | --- | --- |
| Establishment of Site | Cuts/Abrasions Slips/Trips/Falls Manual Handling | 3 | Correct manual handling techniques to be followed. Wear Safety Boots |  |
| Excavation | Contact with overhead or underground services resulting in electrocution/Electric Shock, Fire/ Explosion | 1 | Underground Essential Services to be located prior to commencement of work. Dial-before-you-dig 1100 or www.dialbeforeyoudig.com.au Use location equipment to locate buried services Work only within safe distances of power lines as published by the local supply authority. Isolate any services (electricity, gas) before excavation commences No work is to be carried out on live services. If service is unable to be isolated contact the relevant supply authority Check stability of ground before positioning machinery (including vehicles) Refer to SWMS High Risk Construction – Excavation of 1.5 meter or greater |  |
| Precast septic tank Installation and maintenance Precast Grease traps Oil arrester and other pre-treatment systems installation and maintenance | Manual handling Confined Spaces | 1 | Use safe lifting techniques Regularly check slings use qualified (ticketed) dogger or crane driver for heavy load objects Load to be slung correctly Refer to SWMS High Risk Construction – Confined Spaces |  |
| Task | Risk | Risk Score | Control | Person Responsible |
| Installing and or repairing pipe work from sewer main to and within building | Manual Handling Injury Cuts/Abrasions Slips/trips/falls Burns | 2 | Correct manual handling techniques to be followed. Wear leather gloves. Refer to SDS for priming fluid/solvent cement. Work in a ventilated area |  |
| Test water supply for leaks | Slips/Trips/Falls | 3 | Mop up all spills Wear Safety Boots |  |
| Reinstatement of excavation | Slips/Trips/Falls | 2 | Ground is to be fully reinstated after completion of excavation works. All excess material to be disposed of correctly |  |

| This SWMS has been developed through consultation with our workers and has been read, understood and signed by all workers undertaking the works: |  |  |
| --- | --- | --- |
| Print Names: | Signatures: | Dates: |

| Review No | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial: |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |